Bare Bones documentation of CM and MINQ

K. T. Kilty

4-21-99

CM is a program meant to make a file of move absolute (MA) and plot

absolute (PA) commands. These are equivalent to moves with the pen up and

pen down on a vector plotting device. The vectors include all of the

strokes to label contours and so forth. You can probably use a program

like Golden Software's PLOTCALL to make a hard copy from this file, but

PLOTCALL will not understand any of the header label that is prefixed

to the vector commands. It was never my intention to make a contouring

program, but rather a set of subroutines that could be linked to other

graphics packages. CM is based on some NASA software written many years

ago. It draws contours by an unusual algorithm and serves as an alternative

to the many spline under tension type of algorithms.

CM draws its instruction from a command file which is passed to CM as a

command line parameter. View the file CM2.CMD as an example of a command

file. The command line for CM consists of the name of the command file

and possibly a styring of options. The only options are D=place debug

information on screen while writing vectors into plot file, and S=make

vector commands for stroked fonts. This option will make vector calls

to label contours and so forth. Not using the 'S' option produces a

plot file that uses defined fonts of the plotting package to produce

the labels.

example command line...

c:>cm cm2.cmd SD

will cause CM to open the file cm2.cmd for its run instructions, place

written comments on screen as the plotting proceeds, and build labels

with stroked fonts. Plot files with the 'S' option are larger.

/* key words for command definition file */

"first row",         lower most row of grid

"last row",          upper most row of grid                      /* 1 */

"first column",      left most column of grid                    /* 2 */

"last column",       right most column of grid                   /* 3 */

"grid file",         name of the input grid file                 /* 4 */

"minimum contour",   first contour value                         /* 5 */

"maximum contour",   last contour value                          /* 6 */

"contour interval",  contour interval                            /* 7 */

"level file",        file name containing specific contours      /* 8 */

"label freq",        how often to label contours(1=every one)    /* 9 */

"contour color",     pen to use for contours                     /* 10 */

"contour thickness", how thick to make labeled contours          /* 11 */

"label style",                     /* 12 */

"unlabel color",     pen to use for non-labeled contours         /* 13 */

"unlabel thickness", weight of non-labeled contours              /* 14 */

"unlabel style",                   /* 15 */

"grid subinterval",  how many contour segments across a grid cell /* 16 */

"center type",       how to locate high/low centers               /* 17 */

"center height",     how large to make the marks                  /* 18 */

"hachure length",      how long to make hachure marks             /* 19 */

"hachure suppression", factor to suppress hachures in tight curves/* 20 */

"hachure separation",  how far apart to place hachures            /* 21 */

"pole suppression",    factor to suppress contours around poles   /* 22 */

"contour suppression", factor to suppress contours close together /* 23 */

"inline label color",  pen to use for labels                      /* 24 */

"inline label format",  format for labels (f=fixed)               /* 25 */

"inline decimal digits", how many decimal digits to include       /* 26 */

"inline height",       how large to make contour labels           /* 27 */

"inline tension",                  /* 28 */

"post file",           name of file of original data values       /* 29 */

"symbol angle",        rotation of data locations symbols         /* 30 */

"symbol height",       how large to make data location symbols    /* 31 */

"symbol color",        pen to use for symbols                     /* 32 */

"post label format",   format of data value label (f=fixed)       /* 33 */

"post digits",         how many decimal digits to include         /* 34 */

"post angle",          rotation of data label (0=none)            /* 35 */

"post height",         how large to make data value labels        /* 36 */

"post color",         pen to use for posting data values           /* 37 */

"plot file",          name of file containing vector draws         /* 38 */

"plot scale",         scale factor                                 /* 39 */

"header",             keyword to start an information header       /* 40 */

Here are the default values.

Structure for default values (CONTOURDEF)

cm.dbug=0;                   1=print extra output for error analysis

cm.output=1;  /* TRUE value means that stroked font output is default */

cm.data=NULL;          filename for data points to post.

cm.plotfp=NULL;        filename for plot command output.

cm.gridfp=NULL;        filename of grid for drawing contours

cm.postfp=NULL;

cm.levelfp=NULL;       filename of specific levels to contour

cm.ncols_in_grid=0;    total no. of columns in grid file

cm.nrows_in_grid=0;    total no. of rows in grid file

cm.first_row=0;        index no of first row to contour (bottom of map)

cm.last_row=0;         index no of last row to contour (top of map)

cm.first_column=0;     index to first column to contour (left side)

cm.last_column=0;      index to last column to contour (right side)

cm.pscale=1.0;         plot scale

cm.x_grid_minimum=0.0; world coordinate of grid left

cm.x_grid_maximum=0.0; world coordinate of grid right

cm.y_grid_minimum=0.0; world coordinate of grid lower

cm.y_grid_maximum=0.0; world coordinate of grid upper

cm.x_incr=0.0;         increment between grid nodes left-right

cm.y_incr=0.0;         increment between grid nodes up-down

cm.z_minimum=0.0;      minimum and maximum heights found in grid file

cm.z_maximum=0.0;

cm.minimum_contour=0.0;  minimum contour level (start)

cm.maximum_contour=0.0;  maximum contour level (end)

cm.contour_interval=1.0; contour interval if uniform interval

cm.contour=NULL;

cm.n_cvals=0;            how many contours to draw

cm.center_labl_height=0.0;  how large to make '+' labels for centers

cm.center_labl_style='\0';

cm.hachure_len=0.0;      length if hachures are added to contours

cm.hachure_sep=0.1;      distance between said hachures

cm.hachure_sup=1.01;     factor to suppress hachures in tight curves

cm.poles=5;              factor to suppress bullseye contours around poles

cm.grid_interval=2;      how many segments of contour across each grid cell

cm.labl_line_frequency=1;  how often to label contours

cm.labl_line_color=0;      what color to use for labelled contours

cm.labl_thickness=0.0;     how thick to make labelled contours

cm.labl_style='S';

cm.unlabl_line_color=0;  color for unlabelled contours

cm.unlabl_thickness=0.0; thickness of unlabelled contours

cm.unlabl_style='S';

cm.contour_sup=1.01;    factor to suppress contours in tight places

cm.con_labl_format='f'; label format (fixed=f)

cm.con_labl_dec_digits=0;  how many digits to add beyong decimal

cm.con_labl_height=0.0;  how large to make contour labels

cm.con_labl_color=0;     color of contour labels

cm.con_labl_tension=1.01;  factor to suppress labels along tight curves

cm.post_angle=0.0;       angle to rotate posted symbols (0=left-right)

cm.post_height=0.0;      height of posted symbols

cm.post_color=0;         color of posted symbols

cm.post_labl_format='f'; format of posted labels (f=fixed)

cm.post_dec_digits=0;    decimal digits in posting value

cm.post_labl_angle=0.0;  angle to label the posted value

cm.post_labl_height=0.0; how large to make the posted value

cm.post_labl_color=0;   color of posting labels

cm.vector_size=0;       number of segments to calculate in one sweep

Error returns from job

Terminate Job on any of the following:

exit 1 - no command line parameters.

exit 2 - can't open job file.

exit 3 - not enough memory in far heap.

exit 4 - can't open grid file.

exit 5 - can't open post file.

exit 6 - can't open level file.

exit 7 - abnormal termination kover<0.

exit 8 - abnormal termination unspecified.

exit 9 - not enough memory in near heap.

exit 10- Too many contours

MINQ

If you plan to make contours, you must have a program to grid. This program

makes very smooth grid values and very smooth contours. It works like CM in

that it reads a command file for its instructions. The command line format

is...

c:>minq minq.cmd

MINQ will read the file minq.cmd and interpret each line as an instruction.

MINQ begins with a very coarse grid and cuts the grid dimensions by half

at each iteration stage. Thus, having a grid that is evenly divisible helps

the program run very fast. The best choice is to ask for the numbers of

rows and columns to be a power of 2 plus 1. Thus, 5,9,17,33,65,... are

all good choices. Choosing numbers that are a power of 2 times some prime

number plus one works pretty well also. Thus, 25,49,... work pretty well.

Poor choices can take a long time to grid.

"nrows"   the number of rows to make in the grid file (power of 2 plus 1)

"ncols"   the number of columns in the final grid (power of 2 plus 1)

"xmin"    the left most coordinate value of the grid

"xmax"    the right most coordinate of the grid

"ymin"    the lower coordinate of the grid

"ymax"    the upper coordinate of the grid

"delta"   a convergence factor. Iteration continues until changes no greater

"maxit"   maximum number of iterations to allow

"option"

"data"    data file

"grid"    grid (output) file

"header"  keyword to begin informational header

Lots of grid, command, and plot files eventually become a mess to organize,

so I have made an option for writing header information into each of these

files. The header information has no effect on program operation, but must be

contained between lines "header" and "end header" in the files. Lower case!!